*****************************************************************************

*

* NOTE: o NEVER program security fuse's if your using a ceramic windowed

*         EPROM version of the PIC 16C65,73,74 microcontroller !. The fuse's

*         can't be erased by UV light, so it's an expensive windowed OTP

*         version.

*

*       o In-circuit programming requires the OSC1/CLKIN pin be grounded

*         this in order to avoid problems with the internal program counter.

*

******************************************************************************

VERSION 1.0

pic65.c : A serial mode PIC16C65,73,74 programmer

Features:

o  Auto detects the hex object file formats, INHX16 or INHX8M.

o  Skip unused memory locations and 0x3FFF data bytes during

Programming mode.

o  Features true PC speed independent 100uS programming puls.

o  Embedded fuse information in object file is supported.

o  Fuse information is saved in object file, if PIC16C65,73,74 is read.

o  Auto programming mode, first the PIC is blankchecked then programmed

and finally a verify is performed.

o  Features Read/Dump mode.

Notes/restrictions/bugs:

o  Software needs a IBM compliant computer.

o  This program needs external hardware to be attached

to a parallel port (see pp.asc for an ASCII schematic).

o  Don't use from Windows; another program could grab the LPT port.

o  Run pic65, pic65 -s -2 or pic65 -s -3 to initialise the LPT port.

o  Run pic65, pic65 -t -2 or pic65 -t -3 to test computer and hardware.

o  This is NOT a production quality programmer. (No surprise!).

Revision history:

17-Apr-1998: V-1.0 ; Modified the code to support the PIC16C65,73,74.

Revision history for the pic74pgm.exe programmer.

___________________________________________________________________________

19-Jan-1997: V-1.0 ; if no bug reports are received by the end of this year.

30-Sep-1996: V-1.0b; Software and source code released to the internet.

; Blank Checking bug removed.

20-Sep-1996: V-0.1b: Added auto detect object file format INHX8M or INHX16.

30-Aug-1996: V-0.1b; Added PIT hardware detect and timing correction.

18-Aug-1996: V-0.1b; Modified the 100uS delay, added timeout.

10-Aug-1996: V-0.1b; Removed a few bugs, thanks to Ed's help !.

??-Aug-1996: V-0.1b; First code to successfully program a 16C74.

??-Aug-1996: V-0.0;  Wrote the main code in just one day !.

If you find any bugs please let me know my e-mail vollebre@worldonline.nl.

This version (V-1.0) is no longer a public beta release, but it's still

for free.

Usage:

pic65  [-rpbvadst] [ -lxhrwpc238 ]  objfile

Where objfile is the program data in Intel hex format.

Various things, including the fuse details, can be set on the

command line using the switches described below. Superfluous

command line arguments are silently ignored and later switches

take precedence, even if contradictory. The hex object file format

is automatically detected by software, the detected file format

is displayed on the screen. The object file is checked for validity

by checking the file type, address range, checksum and lost of bytes.

Switches (options) are introduced by - (or /) and are encoded as

a sequence of one or more characters from the sets {rpbvadst} and/or

{lxhrwpc238} Switches can be given individually (like -r /2 -p) or

in groups (like -r2p or /rp2 or /2/r/p which are all synonyms).

Switches are recognized anywhere in the command line

(but don't use the DOS-like pic65/r/p/2  form - this should be written

pic65 /r/p/2;  space around switch groups is important).

There are six different mode's of operation:

Modes:

o    d           -  read memory and dump to screen.

o    r           -  read memory and store data in objfile.

o    p           -  program memory and read data from objfile.

o    v           -  verify program memory and data from objfile.

o    b           -  blankcheck program and configuration memory.

o    a           -  blankcheck, program and verify data from objfile.

There are three logical groups of switches: fuses, ports, hex-types.

They can be mixed freely.

Fuses:

o    l, x, h, r  -  LP, XT, HS or RC oscillator (default LP).

o    w           -  enable watchdog timer.

o    p           -  enable power-up timer.

o    c           -  enable code protection.

Ports:

o   2, 3         -  hardware on LPT2 or LPT3 (default is LPT1).

Hex-types:

o   8            -  objfiles are Intel Hex-8, i.e. INHX8M

(default is Intel Hex-16, i.e. INHX16)

(The software auto detects the Intel hex type)

E.g., -a -rp28 means use autoprogram mode (blankcheck, program and verify)

set the fuses to use the RC oscillator, no watchdog, use power-up timer,

and don't protect code; assume the hardware is on LPT2 and the objfiles

are in INHX8M format. The software will detect and correct if a different

file type is detected, specifying the file type makes sense if the data is

being saved on disk using one of the two hex formats. The INHX16 hex format

is however prefered and advised above the INHX8M hex format.

Improvements :

o Write a programmer for the 12C5XX family of chips (easy),

ASAP, when I receive the windowed 12C5XX version.

o I�C support using the same hardware (easy),

when I need it.

o Porting the sofware to Window NT (porting to W95 is a waste of time).

??.

o Making the programmer a true production quality programmer.

VCC switching (4.5v-5.5v). (hardware and software modification)

o Add a GUI (waste of time?).

o Lots of other stuff I just don't have time for.

Acknowledgements:

Many thanks to Edwin Gouweloos for trusting my programming skills

and testing beta release 0.1b. You must be really MAD or BLIND if

you trust my programming skills Ed, Especially the first release.

I hope the SYNC-MATE will be a success, Ed !. The SYNC-MATE is worlds

smallest RISC based sync/async wonder box, with super-computer power.

Thanks to David Tait for his idea of the LPT programmer and his

pp.exe program (16C84 programmmer) without it I wouldn't be able to

write this program (many thanks).

The programming algorithms used are taken from the data sheets

DS30228D/E/F (C) 1995/1996 Microchip Technology Incorporated.

Thanks to Arizona Microchip Technology Ltd for making this information

available without restriction.

The PC printer port FAQ by Zhahai Stewart for useful info on

PC parallel ports.

Hardware requirements:

The software is tested using the following hardware:

80386 33 Mhz Portable by Texas Instruments.     (Not quiet Compliant)

80486DX2 66 Mhz SOYO Motherboard 25E2.          (Fully Compliant)

Pentium 133 Mhz ASUS Motherboard P55T2P4.       (Fully Compliant)

Pentium Pro 200 Mhz Intel Motherboard Aurora.   (Fully Compliant)

The software is written in such a way that it's CPU speed independent,

however I made the assumption that the used hardware is IBM compliant.

This means that the PIT timer 1 is set to 15.08 uS (count 18), and that

the timer 1 overflow bit can be found at address 0x61,4. If this isn't

so then a special timeout mechanism ensures that the program puls doesn't

exceed the length specified by MICROCHIP. However before programming a

16C65 it's advised to test your computers hardware by issuing the -t

command. This command will test if your computers hardware is IBM

compliant, if it isn't then the program will try to adjust the hardware

in such a way that it meets the standard. We strongly advice not to use

the programmer if the hardware can't be made compliant, if you do use

the programmer it can/will permanently damage the device.

Legal stuff:

This is copyright software, but I do not seek to profit from

it.  I wrote it in the hope that it will prove useful.  Feel

free to modify the program, but please DO NOT distribute

modified versions.  Instead, send me your modifications and

improvements so that they may, at my discretion, be included in a

future release.

Although I cannot undertake to support this software, I would

welcome bug reports, suggestions or comments which should be

sent to:

Paul Vollebregt

Po Box 65824

The Hague 2506 CE.

The Netherlands, NL.

EUROPE.

e-mail: vollebre@worldonline.nl

Copyright (C) 1996 Paul Vollebregt

This program is free software.  Permission is granted to use,

copy, or redistribute this program so long as it is not sold

for profit.

THIS PROGRAM IS PROVIDED AS IS AND WITHOUT WARRANTY OF ANY KIND,

EITHER EXPRESSED OR IMPLIED.